Project 1 — FreeRTOS PID Motor Controller

Interview Preparation Guide

Kishan Kalariya  |  STM32F411RE  |  FreeRTOS  |  PID Control

Mock Interview Score: 97/100

# Section 1 — Project Overview

What you built:

- Closed loop DC motor speed controller on STM32F411RE Nucleo board

- FreeRTOS manages concurrent UART, PID and encoder tasks

- User sends SET_SPEED 0-100 command via PuTTY terminal

- PID algorithm reads encoder feedback every 10ms

- PWM duty cycle adjusted to maintain requested speed

- L298N motor driver + DC motor with quadrature encoder

# Section 2 — Mock Interview Q&A

| Q1: Describe your project in one paragraph | 8/10 |
| --- | --- |

Model Answer:

I built a closed loop DC motor controller on STM32F411RE Nucleo board using FreeRTOS.

The system accepts SET_SPEED commands via UART from PC terminal. Commands are processed by UART task and passed via Queue to PID task. PID algorithm reads encoder feedback every 10ms and adjusts PWM duty cycle to maintain requested speed accurately.

Hardware includes L298N motor driver and DC motor with quadrature encoder. DMA with IDLE interrupt handles UART reception without CPU involvement.

I chose this project because motor control combines all embedded fundamentals: timers, interrupts, DMA, RTOS and real time control algorithms — all used simultaneously in industry.

| Q2: What is context switching? Task vs Thread? | 9.5/10 |
| --- | --- |

Model Answer:

Task vs Thread:

- In FreeRTOS a task is the basic unit of execution — essentially a thread

- Each task has its own stack, priority and runs independently

- FreeRTOS tasks are threads with no process separation (unlike Linux)

- In Linux: threads share heap/file descriptors within same process

Context Switching:

- Saving current task CPU state (registers, program counter, stack pointer) to its Task Control Block

- Then loading another task saved state to resume execution

Triggered by:

- Task yielding or blocking (vTaskDelay, xQueueReceive)

- Timer tick preemption (every 1ms in our project)

- ISR waking a higher priority task

In my project: when SET_SPEED command arrives via IRQ, PID_Task context saved instantly, UART_Task loaded because priority 3 > priority 2.

| Q3: Why Queue instead of global variable? | 10/10 |
| --- | --- |

Model Answer:

Global variable problems that Queue solves:

- Race conditions: Global variable requires manual mutex protection. Queue handles thread-safe access internally.

- Blocking behavior: With global variable PID task must constantly poll wasting CPU. Queue allows PID task to block and wait, yielding CPU until data arrives.

- Data loss: If global variable updates before PID task reads it, intermediate values are lost. Queue buffers up to 10 commands preventing loss.

- No built-in signaling: Global variable needs extra flag + semaphore. Queue combines storage + signaling in one mechanism.

In my project: UART task writes SET_SPEED 80 to queue. PID task blocks on xQueueReceive — runs only when speed changes. No polling, no mutex, no missed commands.

| Q4: What is DMA and why did you use it? | 10/10 |
| --- | --- |

Model Answer:

DMA = Direct Memory Access — a hardware peripheral that transfers data between peripherals and memory without CPU involvement.

How it works:

- CPU configures DMA: source (UART RX register), destination (RAM buffer), size, transfer mode

- DMA controller moves each byte independently as it arrives

- CPU completely free during entire transfer

- DMA raises interrupt only when buffer full or IDLE detected

Why DMA over interrupt-driven UART in motor controller:

- Interrupt per byte: one interrupt every 87 microseconds at 115200 baud

- Each interrupt delays PID loop and motor PWM updates

- DMA batches reception — CPU stays focused on real-time motor control

- With DMA: CPU interrupted zero times during 14-byte command reception

- PID timing stays precise and consistent — motor control stable

| Q5: What is IDLE interrupt and why use it? | 10/10 |
| --- | --- |

Model Answer:

UART IDLE interrupt triggers when receive line becomes idle for more than one character time — typically after a message ends.

Why used in this project:

- UART receives variable-length commands: SET_SPEED 80 (13 bytes), STOP (4 bytes)

- DMA completion interrupt fires only when FIXED buffer size reached

- Variable length commands never fill fixed buffer reliably

IDLE interrupt solves this:

- When IDLE triggers — message is complete, no more bytes coming

- Calculate exact bytes: received = 32 - DMA_counter

- Process complete command immediately

- Re-arm DMA for next message

Result: No polling, no fixed-length buffers, immediate response. Perfect for command-line UART interfaces.

| Q6: Mutex vs Semaphore — explain with project example | 10/10 |
| --- | --- |

Model Answer:

Semaphore is a signaling mechanism with a counter. Any task can signal (give) it regardless of who waited. Used for counting resources or signaling between tasks.

Mutex is a mutual exclusion lock with strict ownership. Only the task that acquired (took) the mutex can release it. Prevents priority inversion and accidental releases.

In my motor project I used Mutex to protect UART peripheral shared between three tasks: MOTOR_Task, ENCODER_Task and SYSTEM_Task.

Without Mutex:

- Task 1 printing: Motor Sp...

- Task 2 interrupts: RPM:120

- Task 1 continues: eed:50%

- PuTTY shows garbage output

With Mutex — only task holding key can print, others wait. Clean output guaranteed.

I chose Mutex over Semaphore because UART is a shared hardware resource requiring strict ownership. Only the actively printing task should release the UART lock. Timeout of 100ms prevents deadlock if any task crashes while holding lock.

| Q7: What was your biggest technical challenge? | 10/10 |
| --- | --- |

Model Answer:

Challenge 1 — UART DMA Circular RX Response Delay:

- Symptom: Motor only responded every 2-3 commands not every command

- Root cause: DMA fires callbacks only at fixed byte counts (16 and 32). Each command is 14 bytes — buffer needed 2+ commands to trigger

- Wrong fix: Buffer size = 14. Rejected because different commands have different lengths

- Correct fix: UART IDLE interrupt detects line silence after each command. Industry standard solution

Challenge 2 — FreeRTOS SysTick Conflict:

- Symptom: Start message incomplete, tasks not switching

- Root cause: xPortSysTickHandler called before scheduler started

- Fix: Check scheduler state before calling xPortSysTickHandler

Challenge 3 — Motor Not Spinning in Integration:

- Symptom: Motor worked standalone, failed in full integration

- Root cause: Two 5V sources on same rail — Nucleo 5V and L298N 5V regulator fighting each other

- Fix: Remove L298N 5V from circuit. Only Nucleo 5V powers encoder

Lesson: Always test each component isolated first. Never assume standalone working means integration working.

| Q8: How would you add a second motor? | 10/10 |
| --- | --- |

Model Answer:

Tasks to add:

- Motor2_Control_Task (same priority as Motor1, reuse same function with pvParameters)

- Extend UART parser to handle SET_SPEED2 and SET_SPEED_BOTH commands

Implementation:

- xTaskCreate(motorControlTask, Motor1, ..., (void*)&motor1Params)

- xTaskCreate(motorControlTask, Motor2, ..., (void*)&motor2Params)

- Same code, different parameter structs — clean reuse

Resources needing protection:

- Separate timers for each PWM — no mutex needed for timing

- Shared UART for printing — existing Mutex handles this

- Separate Queue per motor — UART task writes to specific motor queue

Critical design decisions:

- Same priority for both motor tasks — round-robin scheduling

- Separate encoders on separate timers — TIM4 and TIM2

- Watchdog must be kicked by both tasks or add monitoring task

Result: Independent real-time control, no blocking between motors.

| Q9: What is priority inversion? Could it occur in your project? | 10/10 |
| --- | --- |

Model Answer:

Priority inversion: A higher-priority task is blocked by a lower-priority task holding a shared mutex, while a medium-priority task preempts the lower one, delaying mutex release indefinitely.

Classic 3-task scenario:

- Low task takes mutex

- High task preempts, tries to take mutex — blocks

- Medium task preempts Low (no mutex needed) — runs indefinitely

- High task never gets mutex — system failure

In my project — possible scenario:

- Lowest: Logging task holds mutex while formatting display string

- Highest: PID task needs mutex for speed data

- Medium: UART task preempts logging — runs indefinitely

- PID task never gets mutex — motor control jitter or failure

Prevention — Priority Inheritance (FreeRTOS Mutex does this automatically):

- When xSemaphoreTake used on Mutex, FreeRTOS temporarily boosts mutex holder to priority of waiting higher task

- Once mutex released, priority drops back to normal

- This is why Mutex not binary Semaphore should be used for resource protection

Alternative: Use Queue for PID task — no blocking, no priority inversion risk.

| Q10: What would you do differently if starting again? | 10/10 |
| --- | --- |

Model Answer:

Area 1 — Development Process:

- Test each peripheral completely in isolation before integration

- Write test checklist for each peripheral before combining

- Encoder worked standalone but failed in integration due to initialization order bug

Area 2 — Code Architecture:

- Use CubeMX from beginning — generates correct project structure automatically

- Manual setup caused many hours of include path debugging

- Separate .c and .h files per module: uart_handler.c, motor_control.c, pid_controller.c

Area 3 — Hardware Design:

- Apply single power source rule from beginning

- Never connect two voltage sources to same rail without isolation

- Add decoupling capacitors on encoder VCC line

Area 4 — PID Implementation:

- Start with P only, verify stability, then add I then D

- Implement Ziegler-Nichols auto-tuning for optimal kp ki kd values

Area 5 — Debug Commands via UART:

- GET_RPM, GET_PID, SET_KP 0.5, GET_STATUS commands

- Makes debugging and demonstration much easier

Overall lesson: Embedded systems requires disciplined bottom-up testing and careful hardware power design.

# Section 3 — Quick Reference Formulas

## PWM Math

CCR = (Speed% × ARR) / 100   where ARR = 799

Examples:

- SET_SPEED 50  →  CCR = (50 × 799) / 100 = 399  →  50% duty

- SET_SPEED 80  →  CCR = (80 × 799) / 100 = 639  →  79.9% duty

- SET_SPEED 100 →  CCR = (100 × 799) / 100 = 799 →  100% duty

## PWM Frequency

F = Clock / (ARR + 1) = 16,000,000 / 800 = 20,000 Hz = 20kHz

One cycle = 50 microseconds — above human hearing threshold

## DMA Byte Count

received = RX_BUFFER_SIZE - DMA_counter

Example: Buffer=32, typed 14 bytes → counter=18 → received = 32-18 = 14

## Encoder Speed

delta = current_count - last_count  (every 10ms)

RPM = (delta × 10 × 60) / PPR   where PPR = pulses per revolution

## PID Formula

output = (kp × error) + (ki × integral) + (kd × derivative)

error = target - actual

integral += error  (accumulated over time)

derivative = error - prev_error

# Section 4 — Hardware Components

| Component | Role | Key Detail |
| --- | --- | --- |
| STM32F411RE | Main MCU | 16MHz HSI, Cortex-M4 |
| TIM3 | PWM generation | 20kHz, PA6, AF2 |
| TIM4 | Encoder counter | PB6/PB7, AF2, TI12 mode |
| USART2 | Serial comms | 115200 baud, PA2/PA3 |
| DMA1 Stream5 | UART RX | Channel 4, Circular mode |
| L298N | Motor driver | 12V input, ENA/IN1/IN2 |
| DC Motor | Output device | With quadrature encoder |
| FreeRTOS | RTOS kernel | heap_4, 10KB heap |

Mock Interview Final Score

97 / 100

You are interview ready. Good luck Kishan!