# Custom AXI IP: 8-Value Mean Finder

FPGA/SoC Project Documentation — PYNQ-Z2 (Zynq-7000), Vivado + Vitis

# 1. What Question Was Asked

Build a custom hardware IP block (an AXI4-Lite peripheral) that can be controlled from software running on the Zynq's ARM processor. The hardware must:

- Accept 8 integer values sent one at a time from software

- Store them, add them together, and divide the sum by 8

- Make the final mean (average) available for software to read back

This is a classic 'hardware acceleration' exercise: instead of the ARM CPU calculating the average in software, dedicated logic on the FPGA fabric does the math, controlled through simple register reads and writes (the AXI4-Lite protocol).

# 2. How It Was Solved — The Idea in Plain Words

Think of the design as two mailboxes and a small robot:

- Mailbox IN (slv_reg0) — software drops one number here at a time

- A doorbell (trigger bit in slv_reg1) — software rings it to say 'a new number is ready'

- The robot (a Finite State Machine, or FSM, written in VHDL) — waits for the doorbell, picks up the number, and stores it

- After 8 numbers are collected, the robot adds them all and divides by 8

- Mailbox OUT (slv_reg2) — the robot puts the final answer here for software to read

The FSM moves through simple named steps (states): IDLE → STORE → SUM → DIVIDE → OUTPUT, one step at a time, once per clock tick.

Division by 8 is done using a bit-shift instead of the '/' operator, because plain integer division is not buildable (synthesizable) directly in VHDL hardware — but dividing by powers of 2 (2, 4, 8, 16 …) using a shift is allowed.

# 3. Register Map

| Register | Direction | Purpose |
| --- | --- | --- |
| slv_reg0 | Write (software → hardware) | Input data — one of the 8 numbers to average |
| slv_reg1 | Write (software → hardware) | Trigger bit (bit 0). A 0→1 pulse tells hardware "new value is ready" |
| slv_reg2 | Read (hardware → software) | Final computed mean, valid after all 8 values are written |
| slv_reg3 | Not used | Reserved / unused in this project |

# 4. Step-by-Step: Building This From Scratch

## 4.1 Create the Vivado Project

- Open Vivado → Create Project. Choose a SHORT folder path (e.g. C:/FPGA_Projects/mean_finder) to avoid Windows' 260-character path limit.

- Select the board: PYNQ-Z2 (or your target Zynq board).

- Set Target Language and Simulator Language to VHDL.

## 4.2 Create the Custom AXI IP

- Tools → Create and Package New IP → Create a new AXI4 peripheral.

- Give it a name (e.g. mvariance8_v1_0). Set Number of Registers = 2 or 4 (this project only needs 3 in use).

- Choose interface type: AXI4-Lite.

- Finish the wizard, then click 'Edit IP' to open it as its own small project.

## 4.3 Add the Hardware Logic (VHDL)

Inside the generated file (…_S00_AXI.vhd), only three things are added — everything else in the template (the AXI protocol handling) is left untouched:

- Add library line: use ieee.std_logic_arith.all; and use ieee.std_logic_unsigned.all;

- Add signal declarations for the FSM states, a loop counter, and a running sum — placed just before the 'begin' keyword.

- Add the FSM as a new VHDL process, pasted at the bottom in the 'Add user logic here' section.

Core FSM logic (simplified):

process (S_AXI_ACLK)
begin
  if rising_edge(S_AXI_ACLK) then
    if S_AXI_ARESETN = '0' then
      state <= IDLE;
    else
      case state is
        when IDLE  => state <= STORE;
        when STORE =>
          if trig = '1' and trig_prev = '0' then
            DataA(count) <= CONV_INTEGER(slv_reg0);
            count <= count + 1;
            if count = 7 then state <= SUM; end if;
          end if;
        when SUM    => msum <= msum + DataA(count); ...
        when DIVIDE => msum <= msum / 8;  -- shift, legal in VHDL
        when OUTPUT => slv_reg2 <= CONV_STD_LOGIC_VECTOR(msum,32);
      end case;
    end if;
  end if;
end process;

Important: remove the register's entry from the software 'write' case statement if that register is meant to be hardware-output-only, otherwise two different processes will try to drive the same wire (see Problem table, Section 6).

## 4.4 Package and Add to Block Design

- Save the file → Package IP tab → Review and Package → Re-Package IP.

- Create a Block Design. Add a 'ZYNQ7 Processing System' block and run Block Automation.

- Add your custom IP block. Run Connection Automation to auto-wire it to the processor through an AXI SmartConnect.

- Validate Design (should say 'Validation Successful').

## 4.5 Build the Bitstream

- Sources panel → right-click the block design → Create HDL Wrapper (let Vivado manage it).

- Flow Navigator → Generate Bitstream (this also runs Synthesis and Implementation automatically).

- File → Export → Export Hardware → check 'Include bitstream' → produces a .xsa file.

## 4.6 Vitis: Platform and Application

- Open Vitis. Use a SHORT workspace path (e.g. C:/VITIS_WORKSPACE).

- File → New → Platform Project → point it at the .xsa file → OS: standalone → Processor: ps7_cortexa9_0 → Build.

- File → New → Application Project → select the platform above → Domain: standalone_ps7_cortexa9_0 → Template: Hello World.

- Replace the generated main.c with the test code (Section 5) and Build.

- Connect the board over USB, open a serial terminal (PuTTY, 115200 baud) → Run As → Launch Hardware.

# 5. Software Test Code (C, runs on the ARM core)

#define IP_BASEADDR  XPAR_<YOUR_IP_NAME>_0_BASEADDR
#define REG0  0x00   // input value
#define REG1  0x04   // trigger bit
#define REG2  0x08   // mean result

      
int vals[8] = {10,20,30,40,50,60,70,80};
for (i=0;i<8;i++){
    Xil_Out32(IP_BASEADDR+REG0, vals[i]);
    Xil_Out32(IP_BASEADDR+REG1, 1);  // pulse trigger high
    usleep(10);
    Xil_Out32(IP_BASEADDR+REG1, 0);  // pulse trigger low
}
u32 mean = Xil_In32(IP_BASEADDR+REG2);
xil_printf("Mean = %d\r\n", mean);

# 6. Problems You Are Likely To Face (and the Fix)

This project looks simple on paper but hides several classic FPGA/embedded pitfalls. Every one below was actually hit while building this project.

| Problem | Fix |
| --- | --- |
| VHDL type conversion error on CONV_INTEGER | Add both 'std_logic_arith' and 'std_logic_unsigned' libraries together — one alone is not enough |
| Simulation shows nothing happening | Wrong file was set as simulation top. Explicitly set your testbench as top in the Sources panel |
| Waveform shows same value repeated in all 8 slots | Software writes were much faster than the hardware clock. Fixed by adding a trigger pulse + edge-detect logic instead of a free-running loop |
| '.xsa' export always fails, even after changing folder | Was never a folder-path problem — bitstream generation had not actually finished (check PROGRESS/STATUS of impl_1 before exporting) |
| [DRC MDRV-1] Multiple Driver Nets on slv_reg1 | Two different processes were both writing to the same register. Removed the software-write case for that register so only the FSM drives it |
| VHDL fix does not seem to take effect after editing | Vivado keeps an old cached 'IP revision' inside the block design. Run report_ip_status, then upgrade_ip + generate_target to pull in the fix |
| Vitis keeps showing old/wrong result after rebuilding hardware | Old Vitis platform/application were still cached. Delete both and recreate them fresh, pointing at the newly exported .xsa |

# 7. Final Verified Result (PuTTY output)

Starting Mean Finder Test...
Wrote value: 110
Wrote value: 200
Wrote value: 230
Wrote value: 140
Wrote value: 250
Wrote value: 360
Wrote value: 170
Wrote value: 980
Computed Mean = 305
TEST PASSED!

# 8. Key Lessons (for Interviews)

- Only one process may ever drive a given signal in hardware — two writers to the same register is an illegal 'multiple driver' design.

- A hardware clock runs millions of times faster than software loops. Always use a trigger + edge-detection handshake, never assume software and hardware are naturally in sync.

- A 'done'/status flag must stay stable long enough for software to see it — clearing it too early is a common timing bug.

- Vivado (and Vitis) cache IP versions and platforms. After editing source code, you must explicitly re-package, upgrade the IP instance in the block design, and rebuild the Vitis platform — editing the file alone is not enough.

- Integer division by non-powers-of-2 cannot be built in VHDL directly; use shifts for /2, /4, /8, /16, etc.