# Custom AXI IP: Zero-Crossing Detector (Hardware Square Root)

FPGA/SoC Project Documentation — PYNQ-Z2 (Zynq-7000), Vivado + Vitis

# 1. What Question Was Asked

Build a custom hardware IP that computes the square root of a constant (√8) using only additions, multiplications, and comparisons — no divide or square-root circuit allowed. The method must be binary search:

- Start with a left bound and a right bound that are known to contain the answer

- Repeatedly test the midpoint of the range, and shrink the range by half each time

- After 16 rounds, the midpoint is accurate enough to be the answer

All numbers must use fixed-point representation (e.g. 'int16.10' — a 16-bit number where the last 10 bits represent the fractional part), since a plain integer cannot represent decimals like 2.828.

# 2. How It Was Solved — The Idea in Plain Words

This is the 'guess the number' game, played in hardware:

- The function checked is f(x) = 8 - x². If f(guess) is negative, the guess was too high. If it is zero or positive, the guess was too low.

- Each round: guess = (left + right) / 2, then check f(guess), then move either the left or right bound to the guess.

- A Finite State Machine (FSM) runs this loop automatically in hardware, cycling through steps: INIT → MULT → SUBTRACT → SHIFT → COMPARE, 16 times in a row.

- Software just supplies the starting left/right bounds, presses a 'start' button (a trigger bit), waits for a 'done' flag, then reads the final answer.

Because all math is fixed-point, multiplying two int16.10 numbers produces an int32.20 result, and results must be shifted back down by 10 bits (dividing by 1024) to return to int16.10 format before being reused.

# 3. Register Map

| Register | Read / Write | Purpose |
| --- | --- | --- |
| slv_reg0 | Write | Initial left search bound (leftv), fixed-point format |
| slv_reg1 | Write | Initial right search bound (rightv), fixed-point format |
| slv_reg2 | Write + Read | Write: bit 0 = start trigger. Read: bit 16 = done flag, bits[15:0] = final answer (newx) |
| slv_reg3 | Read | Last computed function value (newy) — mainly for debugging |

# 4. Step-by-Step: Building This From Scratch

## 4.1 Create the Vivado Project and Package the IP

- Same process as any custom AXI IP project: Create Project (short path) → Tools → Create and Package New IP → Create AXI4 Peripheral, 4 registers, AXI4-Lite interface.

- Open the generated …_S00_AXI.vhd file for editing.

## 4.2 Add the Hardware Logic

- Library line: use ieee.numeric_std.all; is enough — do NOT also add std_logic_arith (they both define a conflicting 'signed' type).

- Declare fixed-point types and FSM signals (leftv, rightv, newx, newy, start/done flags, iteration counter) before the 'begin' keyword.

- Connect the start trigger: zc_start <= slv_reg2(0);

- Update the read-back logic (S_AXI_RDATA mux) so reg2 returns {done flag, newx} and reg3 returns newy — counting bits carefully so each register expression is exactly 32 bits wide.

- Add the FSM process at the bottom, in the 'Add user logic here' section.

Simplified FSM outline:

case zc_state is
  when WAIT_START =>
    if start_pulse then
      done  <= '0';           -- only clear done on a NEW search
      leftv <= slv_reg0; rightv <= slv_reg1;
      state <= INIT;
    end if;
  when INIT      => newx <= (leftv+rightv)/2; state <= MULT;
  when MULT      => newx_sq <= newx*newx;     state <= SUBTRACT;
  when SUBTRACT  => diff <= EIGHT - newx_sq;   state <= SHIFT;
  when SHIFT     => newy <= diff / 1024;       state <= COMPARE;
  when COMPARE   =>
    if newy < 0 then rightv <= newx; else leftv <= newx; end if;
    if iter = 15 then state <= DONE_STATE; else iter<=iter+1; state<=INIT; end if;
  when DONE_STATE => done <= '1'; state <= WAIT_START;
end case;

## 4.3 Package, Build Design, Generate Bitstream

- Save → Package IP → Review and Package → Re-Package IP.

- Add to Block Design alongside the Zynq Processing System, run Connection Automation, Validate Design.

- Create HDL Wrapper → Generate Bitstream → Export Hardware (.xsa, include bitstream).

## 4.4 Vitis: Platform, Application, Test

- Create Platform Project pointing at the .xsa (standalone OS, ps7_cortexa9_0).

- Create Application Project (Hello World template) targeting that platform.

- Paste in the test C code (Section 5), Build, then Run As → Launch Hardware with a serial terminal open (115200 baud).

# 5. Software Test Code (C, runs on the ARM core)

#define IP_BASEADDR XPAR_ZEROCROSS_IP_0_BASEADDR
#define REG0 0x00  // leftv
#define REG1 0x04  // rightv
#define REG2 0x08  // bit0=start(write) | bit16=done, [15:0]=newx (read)

      
Xil_Out32(IP_BASEADDR+REG0, 0);        // left = 0.0
Xil_Out32(IP_BASEADDR+REG1, 3*1024);   // right = 3.0 (fixed point)
Xil_Out32(IP_BASEADDR+REG2, 1);  usleep(100);  Xil_Out32(IP_BASEADDR+REG2, 0);

      
u32 r; int done;
do { r = Xil_In32(IP_BASEADDR+REG2); done = (r>>16)&1; } while(!done);

      
int16_t newx_raw = r & 0xFFFF;
float answer = newx_raw / 1024.0;   // ~2.828 = sqrt(8)
xil_printf("sqrt(8) = %d.%03d\r\n", (int)answer, (int)((answer-(int)answer)*1000));

# 6. Problems You Are Likely To Face (and the Fix)

This project is more advanced than a simple accumulator, and introduces fixed-point math bugs and a subtle status-flag timing bug.

| Problem | Fix |
| --- | --- |
| Synthesis failed with 'multiple conflicting declarations of signed' | Two libraries (std_logic_arith and numeric_std) both define a 'signed' type. Removed std_logic_arith — numeric_std alone is enough for to_signed()/signed() conversions |
| Synthesis failed: 'width mismatch, target has 32 bits, source has 48 bits' | A read-register expression was built with too many concatenated bits. Recounted the bits carefully so the total equals exactly 32 |
| Software polling loop hung forever waiting for 'done' | The FSM set done=1 then immediately went to IDLE, which cleared it back to 0 within nanoseconds — far too fast for software to ever see it |
| (Root cause of the above) done flag cleared automatically every loop | Moved the 'clear done' action so it only happens when a NEW search starts (on the start trigger), not automatically every idle cycle — so done now stays high until the next test |
| .xsa export appears to fail repeatedly | Same lesson as Project 1 — always confirm bitstream generation actually finished (STATUS = write_bitstream Complete!) before exporting hardware |

# 7. Final Verified Result (PuTTY output)

Starting Zero Crossing Test...
Wrote leftv = 0, rightv = 3072
Start pulsed. Waiting for done...
Done detected! reg2 = 0x00010B50
newx (raw fixed-point) = 2896
Approx sqrt(8) = 2.828

Check: 2896 / 1024 = 2.828125, and the true value of √8 = 2.828427… — accurate to 3 decimal places after only 16 iterations.

# 8. Key Lessons (for Interviews)

- Binary search is a simple, hardware-friendly way to compute square roots (or solve any monotonic equation) without needing a divider or sqrt circuit.

- Fixed-point math (e.g. int16.10) lets hardware represent decimals using only integers — multiplying two fixed-point numbers doubles the fractional bit-width, so results must be shifted back down.

- Never include two VHDL libraries that define the same type (e.g. std_logic_arith and numeric_std both define 'signed') — pick one.

- Always count bits carefully when building a register value from several smaller pieces — a width mismatch is one of the most common synthesis errors.

- A 'done' status flag must only be cleared when a new operation actually starts, not automatically on every idle cycle, or software will never be able to see it go high.